# Texas Instruments

DC134-002

# ADC3663EVM Test Procedure

# Rev. A

12/3/2020

- GENERAL

  - PURPOSE

    - The purpose of this document is to provide instructions for testing the ADC3664EVM evaluation board.

  - SCOPE

    - This document provides details of test, safety, quality and disposition of the EVMs mentioned above.  Additional information on the boards may be listed in the Reference Documentation section.

  - REFERENCE DOCUMENTATION

    - TIS# SLWU087 (HSDC Pro User’s Guide)

    - ADC3663EVM-SCH.pdf (Schematic)

    - TSW1400EVM-SCH.pdf

Note: Please do not put references to revision letters of documents in the test procedure. This will only cause extra work when docs change.

  - DEFINITIONS

    - EVM is reference to Evaluation Module, the ADC3663EVM.

- SAFETY

  - General Safety

    - This test must be performed by qualified personnel trained in electronics theory and who understand the risks and hazards of the assembly to be tested.

  - Electrostatic Discharge (ESD)

    - ESD precautions must be followed while handling electronic assemblies while performing this test.

    - Personnel handing ESD sensitive material must be familiar with ESD prevention procedures and be equipped with the appropriate attire listed in the Apparel section of this document.

- QUALITY

  - Boards under test are to be visually examined for physical defects, damages or missing components. Defects are to be separated and disposed of as stated in the Material Disposition section.

  - Test limits are to be followed strictly.  Borderline or sporadic pass/fail are considered a FAIL.

  - In the event that the failure rate is greater than 10% (1 in 10), recorded data for the failed units are to be kept for further investigation.

- APPAREL

  - Electrostatic smock (optional)

  - Ground ESD wrist strap

- EQUIPMENT

  - ADC3663EVM

  - 35XX_L_INT (Interposer Card)

  - TSW1400EVM

  - HSDC Pro version 5.20

  - ADC35XXEVM GUI

  - Three R&S SMA100A Signal Generator or equivalent

  - 1 MHz or 5 MHz bandpass filter.

  - 2 Mini USB cable

  - Two banana cables (other side has clips for test points)

  - Multimeter, Fluke 101 or equivalent.

- EQUIPMENT SETUP

  - Apply jumpers to the ADC3663EVM as shown below.

  - Apply jumpers to the TSW1400EVM as shown below.

| ADC3643EVM |  | TSW1400EVM |  |  |
| --- | --- | --- | --- | --- |
| J16 | 2_3 | Default jumpers |  |  |

  - Connect the data interface (FMC) of the ADC3663EVM (J15) to J4 of the LVDS interposer card (ADC35XX_L_INT)

  - Connect J5 (HSMC connector) of LVDS interposer card to J3 of TSW1400EVM.

  - Connect the mini-USB cable to the TSW1400EVM.

  - Connect, and turn on (SW7), 5V (at least 4A capable supply) to J12 of TSW1400EVM.

  - Connect the mini-USB cable to J17 o the ADC3663EVM.

  - Push button S1 on the ADC3663EVM to reset ADC.

  - Using the multimeter, set measure to volts (DC), verify the following test points have the following voltage levels on the ADC3663EVM:

IOVDD	+1.8VDC +/- 0.1V

AVDD  	+1.8VDC +/- 0.1V

  - Connect an SMA cable between the output of a signal generator and the input of the 5 MHz bandpass filter. Set the frequency of the signal generator to 5 MHz with an amplitude of +10 dBm.

  - Connect an SMA cable between the output of the 5 MHz bandpass filter and the analog input of the ADC3663EVM (J2) for Channel A. For Channel B testing, connect to J5. This is the analog input signal.

  - Connect an SMA cable from signal generator to J9 of ADC3664EVM with the frequency at 65 MHz and signal power of ~10 dBm. This is the sampling clock.

  - Connect an SMA cable from signal generator to J7 of ADC3664EVM with the frequency of 260 MHz and signal power of ~13 dBm. This is the data clock input (DCLKIN).

  - Ensure that both signal generators in steps 6.12 and 6.13 (65 MHz and 260 MHz) are frequency locked (sharing the same reference, usually a 10 MHz reference is available on back panel of signal generator). If this is not done, the FFT data will not be correct.

- PROCEDURE

  - Load FTDI template for the ADC3663EVM by using an FTDI programmer (FT Prog).

  - Provide email address of person performing test, and I will provide access to GUI files

  - Open the ADC35XXEVM GUI (Latte), Start Menu  All Programs  Texas Instruments  ADC35XX GUI 1.0.

    - You will not need to program anything. Just make sure the GUI launched and says “Connected” with a green circle at bottom of GUI.

  - Open HSDC Pro 5.2, Start Menu  All Programs  Texas Instruments  High Speed Data Converter Pro.

  - Click “OK” to connect to the TSW1400EVM.

  - Select the ADC tab at the top of the GUI (should be default selection.

  - Use the Select ADC drop down menu at the top left corner and select “ADC3663_2W_16bit”.

  - When prompted to update the firmware for the ADC, click “Yes” and wait for the firmware to download to the TSW1400EVM.

  - Enter “65M” into the ADC Output Data Rate field at the bottom left corner.

  - Click Capture to capture data from the ADC.

  - Adjust the amplitude of signal generator that is supplying the analog input signal so that the value of the fundamental in HSDC Pro is between -1.0 and -1.2 dBFs.

  - Observe the values for the following parameters:

SNR > 81 dB

SFDR > 80 dB

  - Save the captured screen shot to a file.

  - Perform the same steps for Channel B. Connect analog input to J5. Change view to Channel 2/2

  - Loop back to 7.1 and continue until all units have been tested.

- EQUIPMENT SHUTDOWN

  - Power down all supplies and generators.

  - Disconnect all cables.

  - Stow all equipment and cables in prescribed location.

  - Place the TSW1400EVM in an ESD bag.

- MATERIAL DISPOSITION & TRANSFER

  - CONFORMING MATERIAL

On the ESD bag, in text clearly visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled per table below and shipped per the P.O.

| Label 1 Assembly Number+Dash Number (if Applicable) | Label 2 IC Number |
| --- | --- |
| DC134-002 | ADC3663EVM |

  - NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches, or exceeds 5%, contact EVM coordinator for assistance.

Change History:

| Date | Changes | Reason | By |
| --- | --- | --- | --- |
| 12/3/2020 | Original |  | Daniel Brock |